USING YOUR TARYAG LASER CARTRIDGE 
TO  ADJUST IRON OR LASER SIGHTS

Adjusting sites with our laser cartridge is nearly the same as doing it with live ammo – which is how it was always done in the past. But, with the Taryag Laser Cartridge, you can do it just as well at home. Sights are typically adjusted for about 10 yards distance

First, load your laser cartridge. Then set up a target or pick out a spot on the wall. Next, simply take aim with your existing sites and fire a shot from the laser. You don’t even need to mark the hit on the target; simply adjust your sites or laser in the necessary direction to match the laser cartridge. Repeat the process a few times as needed and, when the laser or iron sights point exactly where the laser cartridge is hitting, they’re properly adjusted.

The laser only flashes for about 10 milliseconds, but you’ll have no problem remembering the hit location – it imprints on your retina for several seconds. If the laser stayed on any longer, the dot would appear as a streak, so it would be impossible to identify the hit location.

Laser cartridges are sometimes called “bore sighters”. But, “bore sighters” more accurately refer to the laser devices that are inserted into the muzzle of a rifle. These are typically too long to fit a short barreled revolver or semi-auto. Also, the rifling at the muzzle crown can sometimes interfere with the proper seating of the device. Revolvers present an additional problem – please read our .38 Special Note on our Pistol Cartridge posting.